# Module 4 Study Notes

Troubleshooting PC Hardware — CompTIA A+ Core 1 (220-1201)

# 4.1 — BIOS and UEFI

## 4.1.1 — BIOS and UEFI

Firmware = code in flash memory that initializes hardware before the OS loads. BIOS = legacy, text/keyboard-only. UEFI = modern, GUI + mouse support.

- UEFI advantages: 64-bit support, GUI/mouse, network boot, better boot security, can still fall back to legacy BIOS mode

- Setup access key varies by vendor: Esc/Del/F1/F2/F10/F12 — press during the vendor logo screen

- Missed the key prompt? Shift-click Restart from the Windows logon screen → reaches UEFI boot options

- Legacy BIOS nav: arrow keys, Esc = back, exit choices = save vs discard changes, can reload defaults

- UEFI = mouse-supported GUI, but advanced menus may still need keyboard

## 4.1.3 — Boot and Device Options

Boot order = the sequence firmware searches devices for a bootable OS/boot manager.

- Fixed disk: SATA → lowest-numbered port for boot drive; NVMe (M.2/PCIe) = faster, common modern default

- Optical drive: set top priority only temporarily for install/repair from disc

- USB: needs to be formatted as bootable; used for OS installs & recovery tools

- Network/PXE: boots via network adapter from a configured server — common for enterprise imaging

- Wrong boot device symptom → check boot order first

## 4.1.4 — USB Permissions

Firmware can enable/disable controllers/adapters at the hardware level, including USB — a security control against unauthorized devices, malware, and data exfiltration.

- Can control individual ports or all USB ports depending on the setup program

- Example security tab options: USB Interface, External Ports, Bluetooth, CMOS Camera, Card Reader — each Lock/Unlock

- ⚠ Locking is often all-or-nothing per option (e.g., 'Locked' disables ALL USB devices)

- Enterprise use: locking USB at the firmware level is OS-independent and can't easily be bypassed

## 4.1.5 — Fan Considerations

Clean fans regularly — dust obstructs airflow and causes overheating. Balance intake/exhaust airflow; avoid pushing hot air downward.

- Fan modes: Balanced / Cool / Quiet / Fanless / Custom — found under Cooling/Power/Advanced in firmware

- Custom mode = manual, personalized fan speed control

- Minimum temp threshold = when fans kick in; Duty cycle % = fan speed (higher % = faster)

- Firmware displays live temps from sensors near each fan connector

- Temp monitoring: manually via BIOS/UEFI (F2/Del/F12/Esc) OR third-party OS-level apps

## 4.1.6 — Boot Passwords and Secure Boot

- Supervisor/Setup password = protects BIOS/UEFI settings access

- User/System password = locks the whole system pre-boot; must be shared with all users → weakens security; best for non-interactive-logon machines only

- Secure Boot = UEFI-only feature; verifies bootloader is signed/trusted via cryptographic keys before allowing boot

- Modified/unsigned bootloader = blocked → protects against boot-level malware

- Pre-loaded keys: Microsoft, Fedora, openSUSE, Ubuntu; keys can be added/removed; Secure Boot can be disabled

- Modern systems (e.g., Windows 11) often require UEFI + Secure Boot enabled

## 4.1.8 — Trusted Platform Modules

Encryption = reversible with the correct key (confidentiality). Hashing = one-way, verifies integrity, not reversible.

- TPM = hardware chip storing certs/keys/hashed passwords; has a unique unchangeable endorsement key = root of trust

- TPM checks hashes of firmware/bootloader/OS kernel at boot to detect tampering

- TPM keys are isolated from OS/apps, tamper-resistant, harder to extract than software-stored keys

- BitLocker = real-world example using TPM for key storage

- TPM managed via BIOS/UEFI (enable/disable/reset) and from within the OS

- HSM = removable USB device storing crypto keys; portable alternative/backup to TPM (no TPM support, TPM damaged, or moving an encrypted disk to a new machine)

- HSM requires authentication (password/PIN/fingerprint) to access stored keys

# 4.2 — Power and Disk Issues

## 4.2.1 — Troubleshoot Power Issues

PSU boot sequence: AC→DC conversion → 12V first (spins fans/disks) → tests 5V/3.3V → sends 'power good' signal to CPU.

- No-power diagnosis starts with: LEDs lit? Fans audible?

- Isolation order: check other equipment → test wall socket → verify PSU connections → try another cable/fuse → disconnect extra devices → test PSU with multimeter/tester

- Disconnecting extra devices fixes it → PSU underpowered OR one device is faulty

- ⚠ PSUs are NOT user-serviceable — never open the case

- Suspected bad PSU → don't leave powered on unattended; watch for smoke/fire/smell/noise → shut off immediately

- No cause found after full isolation → likely faulty motherboard or PSU

## 4.2.3 — Troubleshoot POST Issues

POST = firmware diagnostic run after the power good signal, before OS load. Modern PCs boot fast/silently — logo screen shown, messages only on error.

- Power present + fans spinning + blank screen + no beeps → check: what changed (firmware update?) → reset → cabling/adapter seating → faulty device isolation → PSU (fans ≠ power good signal necessarily sent) → CPU/firmware fault

- Jumpers misconfigured after service = possible silent boot blocker

- Beep codes are manufacturer-specific; verify against vendor docs

- Key beep meanings: No beep = PSU/motherboard/speaker fault; Continuous = memory issue; 1 long+1 short = motherboard; 1 long+2-3 short = video adapter; 3 long = stuck key

## 4.2.5 — Troubleshoot Boot Issues

After POST: system checks boot devices in configured order; no bootable device found → error, boot halts.

- Wrong boot device → check for interfering removable media (USB/disc) + verify boot order

- Fixed disk not detected checklist: 1) Power (LED, spin-up, connector) 2) Data cable (damage, both ends connected) 3) UEFI/BIOS (drive enabled? SATA mode AHCI vs RAID correct?) 4) M.2/NVMe (properly seated + detected in firmware)

- Troubleshoot in layered order: physical/power → cabling → firmware detection → firmware config

## 4.2.7 — Troubleshoot Boot Sector Issues

Boot sector issues are suspected only after power/cabling are ruled out. Corruption causes: disk faults, power failure, bad multi-OS install, malware.

- MBR: first sector only, single point of failure, one active primary partition, uses BCD (Windows) or GRUB/LILO (Linux)

- GPT: not single-sector limited, more robust/flexible, more reliable partition identification

- Boot sector corruption error messages: 'Boot device not found,' 'OS not found,' 'Invalid drive specification'

- Blank screen ≠ automatically boot sector — check display cable/connection first

- Malware-caused → antivirus boot disk (scan + repair boot sector); no recovery disk → use OS setup disk's built-in repair options

## 4.2.8 — Troubleshoot OS Errors and Crash Screens

After boot sector loads, errors shift from hardware-focused to mostly software/driver-focused.

- BSOD = Windows-only crash screen; indicates memory faults, driver/device problems, or file corruption

- BSOD causes: bad/incompatible drivers, corrupted system files, defective hardware, overheating/power issues

- Troubleshoot BSOD: scan on-screen QR code → check System log 'BugCheck' entry, note first hex value and search online → memory dump available if under support contract

- macOS equivalent = spinning pinwheel; Linux equivalent = kernel panic / 'Something has gone wrong'

## 4.2.9 — Troubleshoot Drive Availability

HDD failure: early (defect) or late (wear) — mechanical. SSD failure: write-cycle wear on memory cells — no moving parts.

- Grinding/clicking/scraping noise = HDD mechanical failure (doesn't apply to SSD)

- No LED activity = power/connection issue (single drive) or missing/failed array (RAID)

- Constant LED activity = low RAM (paging), bad software/malware, OR failing disk

- Bootable device not found = file corruption, faulty drive, OR RAID controller not detecting array member(s)

- Missing in OS = check init/partition/format first; not detected at all = hardware/cable fault

- Read/write failure ('cannot read from source disk') = bad sectors (HDD)/bad blocks (SSD) → run chkdsk, increasing count = imminent failure

- Any of these symptoms → back up immediately + replace drive

## 4.2.11 — Troubleshoot Drive Reliability and Performance

S.M.A.R.T. = built-in disk self-diagnostic, can proactively alert the OS to failure.

- Run diagnostics via vendor utilities, bundled system diagnostics, or Windows/third-party SMART tools

- Tests report damage + performance stats like IOPS

- Metrics below baseline = drive likely faulty; metrics normal = look elsewhere (app load, fragmentation [HDD only], low free space)

- Bad sectors (HDD) / bad blocks (SSD) → firmware marks them unusable going forward

- ⚠ SSD file recovery is generally not possible without specialized tools (unlike HDD)

## 4.2.12 — Troubleshoot RAID Failure

RAID protects against single-disk failure via mirroring or parity; presented to the OS as one volume.

- Device failure (single disk) → volume shows 'degraded,' data still accessible, still bootable if configured

- RAID 0 = NO redundancy — one disk fails, whole volume dies; RAID 0 = speed-focused, not reliability-focused

- Array failure = most desktop RAID tolerates only 1 lost disk; replace ASAP

- Hot swap supported → insert new disk → rebuild via RAID config utility (hardware) or OS utility (software); rebuild = heavy temporary performance hit

- ⚠ Never hot-swap a healthy disk — check for red LED (failure indicator); always back up before swapping

- 'Array missing'/unavailable volume = too many failed disks OR controller failure; boot volume affected = OS won't start

- Controller failure = data usually recoverable, possible corruption from interrupted writes; fix = new controller or move disks to another system

- Can't access RAID config utility at all = likely controller failure itself

## 4.2.13 — Real-World Storage/RAID Troubleshooting (Industry Interview)

- Real-world SSD issues often stem from misconfiguration (mixing different SSD models/specs), not hardware failure

- Spinning HDDs vulnerable to heat, temperature, vibration; audible whine + orange/red LED = failure warning signs

- Common modern spinning-disk errors: 'target not found,' data corruption — increasing as disks age

- SSD diagnostic approach: Disk Management/BIOS first (confirm system sees the disk) → Performance Monitor → vendor/proprietary software (config + firmware)

# 4.3 — System and Display Issues

## 4.3.1 — Troubleshoot Component Issues

Lockups/shutdowns/crashes → often software/corruption/malware, not hardware — rule that out first.

- Pattern-based errors (e.g., after running a while) → suggests a thermal issue

- Check PSU for stable voltage before suspecting other hardware

- Vendor hardware diagnostics often run from firmware, not OS

- No diagnostic tools available → fall back to physical/visual inspection

- RAM: needs firm, EVEN pressure on both sides to seat correctly — most 'failure to POST' RAM issues are installation issues, not dead sticks

- CPU: needs minimal/no force to seat — real pressure needed = something's misaligned, stop and recheck; verify seating BEFORE attaching the heat sink

- Fan whine = end-of-life, bad power connection, or physical interference (cable rubbing) — reseat/reconnect first

## 4.3.2 — Overheating

Hot to the touch = check for overheating; burning smell/smoke = likely PSU, shut down immediately. Dust-clogged vents can also cause a burning smell without actual failure.

- Diagnostic techniques: temp sensors (compare to vendor docs), CPU fan (connected/clogged/undersized), heat sink (snug fit, fresh thermal paste), blanking plates (cover unused case holes), environment (room temp/dust, avoid radiators/direct sun)

- A fan sized for an old CPU may be inadequate after a CPU upgrade — check cooling capacity, not just fit

- Thermal stress side effects: loosened connectors, shifted components, widening hairline PCB cracks — some visually detectable

## 4.3.3 — Physical Damage

Physical damage most often hits peripherals/ports/cables; internal damage is more likely after transit. Even small case cracks/dents can signal a fall causing hidden internal damage.

- Peripheral not working → check port/cable pins (bent/broken/dirty) + full cable length

- Motherboard damage causes: ESD/electrical spikes/overheating (chip damage), careless insertion (bent pins), dirt/chip creep (adapter loosens over time from thermal cycling)

- Visible damage signs: liquid spills, dust clogging, scorch marks (blown component), swollen/bulging capacitors

- Suspected motherboard damage → use diagnostic software to confirm; known-good component swapping is too costly/time-consuming as a primary method

## 4.3.4 — Troubleshoot Performance Issues

Structured order: check overheating/throttling → check misconfigurations (esp. after recent changes) → verify/isolate via diagnostics → rule out software/config/network causes.

- Recent build/upgrade/maintenance → check component compatibility; a memory upgrade can accidentally break dual-channel mode

- Standing diagnostic question: 'What has changed?'

- Diagnostics should isolate compute vs storage vs networking — compare against known baselines, quantify 'sluggish'

- Bottleneck = weakest component limits whole-system performance regardless of other strong specs (classic: fast CPU + HDD = still sluggish)

- Modern bottlenecks: NVMe misconfiguration, latency, PCIe lane/bandwidth mismatch (e.g., PCIe 3.0 vs 4.0)

- Rule out software/config BEFORE assuming hardware — e.g., a slow login script mimicking 'slow computer'

## 4.3.5 — Troubleshoot Inaccurate System Date/Time

Wrong system date/time disrupts network authentication and scheduled tasks (backups, etc.).

- RTC = component tracking date/time; powered by a CR2032 coin-cell battery when the system is off

- Time wrong in system setup itself → likely failing RTC battery → replace with same type

- 'CMOS battery' = legacy name; modern systems use NVRAM/flash for config data — battery today mainly just sustains the RTC

- NTP = modern systems auto-sync time over the network, reducing reliance on RTC battery accuracy day-to-day

## 4.3.6 — Troubleshoot Missing Video Issues

- No image: check power/standby first → then input source (via OSD menu)

- Cabling: check both ends secure, not damaged; confirm cable spec matches use case (e.g., High-Speed HDMI for 4K)

- Rule out cable fault: swap in a known-good cable, OR test the monitor on a different PC to isolate display vs source

- Projectors use CRT/LCD/DLP + high-intensity bulbs (vs monitor backlights/LEDs)

- Failing bulb signs: dimming, bulb health warning; burnt-out bulb: popping sound, scorch marks, broken filament

- ⚠ Let a projector fully cool before handling/replacing the bulb

- Intermittent projector shutdown → overheating is the #1 suspect (fan, vents/dust, ambient temp); if not, check loose cables, bulb seating, firmware updates

## 4.3.7 — Troubleshoot Video Quality Issues

- Dim → check OSD brightness/power-saving features first; unresponsive dimness = backlight failure

- Fuzzy → resolution mismatch (output vs native) → fix in OS display settings or update driver

- Flashing → check cable first; could be backlight/circuitry OR overheating/faulty video card → isolate with a different PC

- Stuck pixel = sometimes fixable (cycling software, gentle tap); dead pixel = usually not repairable, check warranty

- Burn-in = static image too long; OLED/plasma more prone than TFT/LED (self-illuminating pixels vs shared backlight); prevent with screensaver or auto-off

- Color calibration = Color Management applet (Control Panel) + test patterns + spectrophotometer; gamma = RGB input vs light output relationship

- Color glitches (colored lines/shifts) → try cable replacement first → persists = monitor/GPU hardware fault

- Audio: HDMI/DisplayPort carry audio; DVI/VGA do NOT

- Sizing issues → match native resolution, use OSD to fit, update video drivers

- Distortion/warping → check interference, cable connections, resolution match; CRT = pincushion adjustment + check cable

## 4.3.8 — Real-World Video Troubleshooting (Industry Interview)

- #1 real-world video issue = loose/incorrect connection, not hardware failure — 'has this worked before?' is a fast diagnostic

- Docking stations: unplug/replug the dock from the PC, not each individual peripheral

- VGA/DVI = screw-secured, sturdier; HDMI/DisplayPort = no lock mechanism, loosens more easily

- After ruling out physical connection: check driver/firmware/OS updates next

- Driver reinstall = ~5 min fix vs ~24 hrs for a full setup replacement — always try the fast fix first

- Uninstall old driver BEFORE installing a new one; Display Driver Uninstaller (DDU) = clean-slate tool for graphics driver resets

- OLED burn-in = resurfacing modern issue, same root cause as legacy displays — fix = screen saver, avoid static 24/7 images

# 4.6 — Additional Resources

## 4.6.1 — BIOS and UEFI (recap)

- BIOS's 2.2TB drive limit exists specifically because of MBR's addressing limits

- UEFI's GPT support is what unlocks booting from drives larger than 2.2TB

## 4.6.2 — Troubleshooting Example: Video Issues (worked case study)

Formal troubleshooting methodology demonstrated: Identify problem → establish theory → test theory (repeat if disproven) → escalate/research if direct testing fails → plan of action → implement fix → verify FULL functionality (not just the original symptom) → document.

- Video failure points: cables (simplest fix), video card (onboard vs expansion, missing power connector), monitor (backlight failure = ghost image, internal PSU), drivers (variable quality by vendor), external cable surge/lightning exposure

- A disproven theory isn't a failure — it's expected; move to the next theory

- Vendor websites/documentation can reveal known compatibility issues not obvious from generic troubleshooting

- Always verify a fix against broader system use, not just the one symptom that triggered the ticket

- Documentation (both from the user and in a ticketing system) speeds up all future troubleshooting

Quiz & Review Key Takeaways (4.1.11, 4.2.14, 4.3.10, 4.7, 4.8)

- Boot order issues → always fix in boot option sequence, not by disabling/disconnecting hardware

- Secure Boot ≠ TPM: Secure Boot blocks untrusted bootloaders at boot; TPM generates/stores keys + verifies hashes

- Unsigned OS (e.g., fresh Linux install) blocked by Secure Boot → disable Secure Boot to allow it to boot

- HSM = TPM's portable alternative when TPM isn't supported — not a performance or backup tool

- 'Quiet' fan mode = reduced speed, higher tolerated temps ≠ 'Fanless' (no active cooling at all)

- Can't access RAID config utility at all = controller failure (not 'array missing,' which means too many disks failed relative to RAID tolerance)

- 'No Bootable Device' → check boot order first, before cables/diagnostics/reinstall

- BSOD hex codes → check event logs matching that subsystem (e.g., NTFS code → disk logs)

- Clicking/grinding noise = mechanical HDD failure signature → replace + restore from backup, not defrag/malware scan

- New hardware not detected → check what you just touched (connectors/seating) before escalating to PSU/motherboard replacement

- No power at all (LEDs off, fans silent) → test wall socket before any internal component work

- Gradual worsening over time (not sudden, not random) = classic dust/thermal buildup signature

- PSU stability = confirmed via multimeter voltage measurement, not visual inspection alone

- 'No signal' (projector/monitor) = cable/connection issue; blank/dim with power on = bulb/backlight issue

- Post-upgrade sluggishness → check the specific thing that changed (dual-channel mode, SSD config) before broader diagnostics

- Auth failures on a connected-but-unauthenticated system → check date/time sync, not cabling/disk/overheating

- CPU premature failure → ESD is the common culprit, not heat (thermal protection exists) or magnetism

- Post-CPU-upgrade shutdown after ~15 min → always check fan power + thermal paste application first

- USB 2.0 = 480 Mbps max (vs USB 1.1's 12 Mbps)

- SD card capacities: SD ≤2GB, SDHC ≤32GB, SDXC ≤2TB

- Optical drive stuck/won't eject → try the manual eject pinhole (paper clip) first

- Servers commonly use redundant, hot-swappable PSUs

- Full troubleshooting sequence: gather info/identify changes → form hypothesis → test → implement fix → back up before implementing when data is at risk → test solution → document

- RAID failure analysis should include root-cause investigation (controller logs), not just replacing the failed drive